EEET2481 Embedded System Design and Implementation

Semester 1, 2025

Assessment 3 Report

# 1. Introduction

We present the development and demonstration of two embedded projects on the Nuvoton M487 development board:

Exercise 1: Rewrite the Tutorial-8 LCD demo.

Exercise 2: Implements a full Tetris game with screen display and (joystick + buttons) for controls.

Github link for exercise 2: https://github.com/truongbaan/NuvoM487Tetris-An-and-Minh

# 2. Exercise 1: Rewriting Tutorial 8

This task involved re-implementing hardware initialization without using predefined parameters and adding new LCD drawing functions.

## Flowchart

# 3. Exercise 2 – Tetris Game on M487 Board

In Exercise 2, we built a classic Tetris game on the M487 board. Key features include:

- Game field: 24×32 units (10×10 pixel blocks), with a 14-unit-wide play area and 7-unit-wide info panel.

- Controls: 4-way joystick moves and rotates pieces; a single push-button (SW1) pauses and advances between game stages; another button (SW2) for hard-drop.

Stages in the game:

- Start Screen: This will present the image of tetris (You can either touch screen to start or use any button to enter the game)

- GamePlay: falling pieces, scoring, level progression.

- Pause Mechanic: freezes time and displays pause. (not really a screen but pause stage)

- Game Over: when blocks reach the top, the game ends and displays Game Over Screen.

- High Score: shows ranked top scores. Press any buttons to return to the GamePlay.

## 3.1 Game Design Summary

Figure 1: Actual Game Screen

Play Field:

- Dark grid of dots (24 rows × 14 columns) framed by a 1-unit-thick border.

- Active pieces are rendered in their unique colors; settled blocks form the pile.

Info Panel:

- NEXT: previews the upcoming piece in its color and orientation.

- SCORE / HISCORE: current score increments by 1 per line cleared; HISCORE persists between runs.

- LEVEL: starts at 1 and increases every 5 lines (max 10), which accelerates piece drop speed.

- TIME: elapsed runtime in mm:ss using Timer 0 at 1 Hz.

- PAUSE: reminder that SW1 toggles pause using an interrupt.

## 3.2 Flowchart and FSM

Figure 2: Tetris Game Logic Flow Chart

## 3.3 Game Mechanics (Functions)

| Function | Category | Purpose / Usage |
| --- | --- | --- |
| SYS_Init | Initialization | Configure system clock, EBI, UART0 pins (MFP), and basic GPIO settings |
| GPIO_Init | Initialization | Set up all GPIO ports/pins (joystick inputs, SW1, LCD control lines) before use. |
| Timer0_Init | Initialization | Configure Timer0 in periodic mode at 1 Hz to track elapsed play time (mm:ss). |
| Timer1_Init | Initialization | Configure Timer1 for the falling‐piece “tick” interrupts, with reload values based on current level (0.5 s → 0.05 s). |
| disableSelectiveInterrupts | Interrupt Management | Temporarily disable specific NVIC interrupt lines while critical sections run |
| enableSelectiveInterrupts | Interrupt Management | Re-enable previously disabled NVIC interrupt lines |
| TMR0_IRQHandler | ISR | Fired every 1 s—increments the played‐time counter and requests a redraw of the time display. |
| TMR1_IRQHandler | ISR | Fired at the current drop interval—sets a flag so the main loop will advance the active piece downward. |
| GPA_IRQHandler | ISR | Handles joystick “UP” (rotate) and “DOWN” (hard drop) button presses on port A |
| GPC_IRQHandler | ISR | Handles joystick “LEFT” and “RIGHT” movement presses on port C. |
| GPG_IRQHandler | ISR | Handles SW1 presses on port G for Pause/Stage‐change events. |
| Default_Handler | ISR Fallback | Catches any unexpected interrupts routed to this default vector. |
| HardFault_Handler | Fault Handler | Low‐level HardFault entry point that captures register state. |
| HardFault_HandlerC | Fault Handler | C routine that logs or handles a HardFault once control has been transferred here. |
| createBoard | Game Logic | Initialize the 2D play‐field array (24×32) to all zeros (empty). |
| printBoard | Rendering | Draw the current play‐field matrix to the LCD, including settled pieces and the active piece. |
| generatePiece | Game Logic | Randomly select one of the seven Tetris shapes, assign it as the “next” piece and prepare the new active piece. |
| canMove | Collision Detection | Check whether the active piece can legally move or rotate to a given position without colliding or exiting the boundary |
| movePieceDown | Movement | For auto moving down and for speed up going down |
| movePieceLeft | Movement | The controlled piece goes left by 1 |
| movePieceRight | Movement | The controlled piece goes right by 1 |
| rotatePiece | Movement | Rotate the active piece 90° clockwise in response to an UP joystick press (with collision correction). |
| computeDropDistance | Game Logic | Calculate how many empty rows lie below the active piece (used by hard‐drop). |
| hardDrop | Movement | Instantly move the active piece to its lowest legal position and settle it into the play‐field |
| removeLines | Game Logic | Scan the play‐field for any full horizontal lines; remove them and shift all above rows down, updating the score |
| isGameOver | Game Logic | Check if any block cell has reached the top row |
| key_input | Input Handling | Central polling function for joystick and SW1 flags; debounces and routes to movement, rotate, pause, or stage‐change actions. |
| insertScore | High Score | Insert the current final score into the leaderboard array in sorted order, capping to the top n entries |
| drawScoreScreen | Rendering | Display the HIGH SCORE stage layout with the sorted leaderboard on the LCD. |
| drawNextPiece | Rendering | Render the “next piece” preview box in the info area |
| drawGameScore | Rendering | Draw the current game score value in the status area |
| drawTopScore | Rendering | Draw the all‐time high score (from previous sessions) in the status area. |
| drawLevel | Rendering | Show the current level (1–10) in the status area |
| drawTime | Rendering | Show the elapsed play time (mm:ss) using Timer0 data |
| drawPause | Rendering | Overlay “PAUSED” indicator and instructions when the game is suspended. |
| drawUnpause | Rendering | Remove the pause overlay and resume normal display |
| drawGameOver | Rendering | Overlay the “GAME OVER” screen and prompt to view high scores |
| gameplay | Core | The main game‐state loop: checks flags (timer, input), moves/rotates pieces, calls collision, line‐clear, rendering, and state transitions |
| main | Starting point | Calls all initializers, then enters the game. (start at game start screen) |

## 3.4 Timing and Waveforms

Each of the key timings for the corresponding timers used in the project were verified using an oscilloscope through the timer interrupt toggling a GPIO (PB.2 in this case) to measure the timing.  Some of the measurements (TIMER0 & TIMER1) were taken over a complete cycle therefore the measurement should be divided by 2 to get the timing of the actual delay generated by the interrupt.

Figure 3: TIMER0 - Game Timer - 1 Second Counter

Figure 4: TIMER1 - Game Speed Delay - Level 1 - 0.5 Second

Figure 5: TIMER1 - Game Speed Delay - Level 2 - 0.45 Second

Figure 6: TIMER1 - Game Speed Delay - Level 3 - 0.4 Second

Figure 7: TIMER1 - Game Speed Delay - Level 4 - 0.35 Second

Figure 8: TIMER1 - Game Speed Delay - Level 5 - 0.3 Second

Figure 9: TIMER1 - Game Speed Delay - Level 6 - 0.25 Second

Figure 10: TIMER1 - Game Speed Delay - Level 7 - 0.2 Second

Figure 11: TIMER1 - Game Speed Delay - Level 8 - 0.15 Second

Figure 12: TIMER1 - Game Speed Delay - Level 9 - 0.1 Second

Figure 13: TIMER0 - Game Speed Delay - Level 10 - 0.05 Second

Figure 14: TIMER2 - Game Piece Spawn Delay - Level 1 - 0.5 Second

Figure 15: TIMER2 - Game Piece Spawn Delay - Level 2 - 0.45 Second

Figure 16: TIMER2 - Game Piece Spawn Delay - Level 3 - 0.4 Second

Figure 17: TIMER2 - Game Piece Spawn Delay - Level 4 - 0.35 Second

Figure 18: TIMER2 - Game Piece Spawn Delay - Level 5 - 0.3 Second

Figure 19: TIMER2 - Game Piece Spawn Delay - Level 6 - 0.25 Second

Figure 20: TIMER2 - Game Piece Spawn Delay - Level 7 - 0.2 Second

Figure 21: TIMER2 - Game Piece Spawn Delay - Level 8 - 0.15 Second

Figure 22: TIMER2 - Game Piece Spawn Delay - Level 9 - 0.1 Second

Figure 23: TIMER2 - Game Piece Spawn Delay - Level 10 - 0.05 Second

# 4. Team Contribution Summary

| Team Member | Responsibility | Contribution Score |
| --- | --- | --- |
| Truong Ba An (s3999568) | Tetris game logic, Game Logic debugging | 50% |
| Minh Duc Vu (s4054648) | LCD draw functions, Tetris game button controls, Hardware & game debugging | 50% |